
Emitted Colour : YELLOW 
Size (mm) : 5mm T1 3/4 
Lens Colour : Water Clear 
Peak Wave Length (nm) : 585 ~ 595 
Forward Voltage (V) : 1.8 ~ 2.2 
Reverse Current (uA) : <=30 
Luminous Intensity Typ Iv (mcd) : Average in 5000 
Life Rating : 100,000 Hours 
Viewing Angle : ±10°
Absolute Maximum Ratings (Ta=25°C)

Max Power Dissipation : 80mw 
Max Continuous Forward Current : 30mA 
Max Peak Forward Current : 75mA 
Reverse Voltage : 5~6V 
Lead Soldering Temperature : 240°C (<5Sec) 
Operating Temperature Range : -25°C ~ +85°C 
Preservative Temperature Range : -30°C ~ +100°C
